- Rpi install

- R pi switch LED

Code:

importRPi.GPIO as GPIO

GPIO.setmode(GPIO.BCM)

GPIO.setup(17, GPIO.OUT)

GPIO.setup(22, GPIO.IN)

while True:

ifGPIO.input(22):

GPIO.output(17, GPIO.HIGH)

else:

GPIO.output(17, GPIO.LOW)

GPIO.cleanup()

  - Temperature sensors(LM35)  with Arduino

Code:

const int lm35_pin = A1;	/* LM35 O/P pin */

void setup() {

Serial.begin(9600);

}

void loop() {

int temp_adc_val;

float temp_val;

temp_adc_val = analogRead(lm35_pin);	/* Read Temperature */

temp_val = (temp_adc_val * 4.88);	/* Convert adc value to equivalent voltage */

temp_val = (temp_val/10);	/* LM35 gives output of 10mv/°C */

Serial.print("Temperature = ");

Serial.print(temp_val);

Serial.print(" Degree Celsius\n");

delay(1000);}

  - Interfacing IR sensors with Arduino

# Code :-

int IRSensor = 9; // connect ir sensor module to Arduino pin 9

int LED = 13; // conect LED to Arduino pin 13

void setup()

{

Serial.begin(115200); // Init Serila at 115200 Baud

Serial.println("Serial Working"); // Test to check if serial is working or not

pinMode(IRSensor, INPUT); // IR Sensor pin INPUT

pinMode(LED, OUTPUT); // LED Pin Output

}

void loop()

{

int sensorStatus = digitalRead(IRSensor); // Set the GPIO as Input

if (sensorStatus == 1) // Check if the pin high or not

{

// if the pin is high turn off the onboard Led

digitalWrite(LED, LOW); // LED LOW

Serial.println("Motion Ended!"); // print Motion Detected! on the serial monitor window

}

else

{

//else turn on the onboard LED

digitalWrite(LED, HIGH); // LED High

Serial.println("Motion Detected!"); // print Motion Ended! on the serial monitor window

}

}

3.3 Interfacing PIR sensors with Arduino

# Code :-

int ledPin = 13;                // choose the pin for the LED

int inputPin = 8;               // choose the input pin (for PIR sensor)

int pirState = LOW;             // we start, assuming no motion detected

int val = 0;                    // variable for reading the pin status

void setup() {

pinMode(ledPin, OUTPUT);      // declare LED as output

pinMode(inputPin, INPUT);     // declare sensor as input

Serial.begin(9600);

}

void loop(){

val = digitalRead(inputPin);  // read input value

if (val == HIGH)	// check if the input is HIGH

{

digitalWrite(ledPin, HIGH);  // turn LED ON

if (pirState == LOW)

{

Serial.println("Motion detected!");	// print on output change

pirState = HIGH;

}

}

else

{

digitalWrite(ledPin, LOW); // turn LED OFF

if (pirState == HIGH)

{

Serial.println("Motion ended!");	// print on output change

pirState = LOW;

}

}

}

4.1 Stepper motor

# Code :-

# importRPi.GPIO as GPIO

from time import sleep

import sys

#assign GPIO pins for motor

motor_channel = (29,31,33,35)

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BOARD)

#for defining more than 1 GPIO channel as input/output use

GPIO.setup(motor_channel, GPIO.OUT)

motor_direction = input('select motor direction a=anticlockwise, c=clockwise: ')

while True:

try:

if(motor_direction == 'c'):

print('motor running clockwise\n')

GPIO.output(motor_channel, (GPIO.HIGH,GPIO.LOW,GPIO.LOW,GPIO.HIGH))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.HIGH,GPIO.HIGH,GPIO.LOW,GPIO.LOW))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.LOW,GPIO.HIGH,GPIO.HIGH,GPIO.LOW))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.LOW,GPIO.LOW,GPIO.HIGH,GPIO.HIGH))

sleep(0.02)

elif(motor_direction == 'a'):

print('motor running anti-clockwise\n')

GPIO.output(motor_channel, (GPIO.HIGH,GPIO.LOW,GPIO.LOW,GPIO.HIGH))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.LOW,GPIO.LOW,GPIO.HIGH,GPIO.HIGH))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.LOW,GPIO.HIGH,GPIO.HIGH,GPIO.LOW))

sleep(0.02)

GPIO.output(motor_channel, (GPIO.HIGH,GPIO.HIGH,GPIO.LOW,GPIO.LOW))

sleep(0.02)

#press ctrl+c for keyboard interrupt except KeyboardInterrupt:

#query for setting motor direction or exit

motor_direction = input('select motor direction a=anticlockwise, c=clockwise or q=exit: ')

#check for exit

if(motor_direction == 'q'):

print('motor stopped')

sys.exit(0)

4.2 Arduino DC motor speed control

Code:

#define enA 9

#define in1 6

#define in2 7

void setup() {

pinMode(enA, OUTPUT);

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

// Set initial rotation direction

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

}

void loop() {

int potValue = analogRead(A0); // Read potentiometer value

int pwmOutput = map(potValue, 0, 1023, 0 , 255);

// Map the potentiometer value from 0 to 255

analogWrite(enA, pwmOutput); // Send PWM signal to L298N Enable pin

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

delay(20);

}

5) MQTT

# write this code in python IDE or directly from terminal of Raspberry PI

# Code :-

# ThingSpeak Update Using MQTT

# Compiled by Ashish Kumar (ashishmarch12@gmail.com) # Date: 07 April 2018

from     future   import print_function import paho.mqtt.publish as publish import psutil

import sys

import Adafruit_DHT import time

# Replace this with your Channel ID

channelID = "XXXXXX"

# The Write API Key for the channel

apiKey = "XXXXXXXXXXXXXXXX"

# MQTT Connection Methods useUnsecuredTCP = True useUnsecuredWebsockets = False useSSLWebsockets = False

# The Hostname of the ThinSpeak MQTT service

mqttHost = "mqtt.thingspeak.com"

# MQTT connection parameters

if useUnsecuredTCP: tTransport = "tcp" tPort = 1883

tTLS = None

if useUnsecuredWebsockets: tTransport = "websockets" tPort = 80

tTLS = None

if useSSLWebsockets: import ssl

tTransport = "websockets"

tTLS = {'ca_certs':"/etc/ssl/certs/ca-certificates.crt",'tls_version':ssl.PROTOCOL_TLSv1} tPort = 443

# Create the topic string

topic = "channels/" + channelID + "/publish/" + apiKey

while(True):

humidity, temperature = Adafruit_DHT.read_retry(11, 4) print("Temperature = %s	Humidity = %s" % (temperature, humidity)) tPayload = "field1=" + str(temperature) + "&field2=" + str(humidity)

time.sleep(10) try:

publish.single(topic, payload=tPayload, hostname=mqttHost, port=tPort, tls=tTLS, transport=tTransport)

except (KeyboardInterrupt):

break

except:

print ("There was an error while publishing the data.")

6) Web server Rpi LED

Below is the complete code for creating two buttons to turn on and off the LED connected to Raspberry Pi.

# Code :-

<html>

<head>

<meta name="viewport" content="width=device-width" />

<title>Raspberry Pi WiFi Controlled LED</title>

</head>

<body>

<center><h1>Control LED using Raspberry Pi Webserver</h1>

<form method="get" action="index.php">

<input type="submit" style = "font-size: 14 pt" value="OFF" name="off">

<input type="submit" style = "font-size: 14 pt" value="ON" name="on">

</form>​​​

</center>

<?php

shell_exec("/usr/local/bin/gpio -g mode 27 out");

if(isset($_GET['off']))

{

echo "LED is off";

shell_exec("/usr/local/bin/gpio -g write 27 0");

}

else if(isset($_GET['on']))

{

echo "LED is on";

shell_exec("/usr/local/bin/gpio -g write 27 1");

}

?>

</body>

</html>

7) Bluetooth

# Code :-

char data = 0;           //Variable for storing received data

void setup()

{

Serial.begin(9600);  //Sets the data rate in bits per second (baud) for serial data transmission

pinMode(13, OUTPUT); //Sets digital pin 13 as output pin

}

void loop()

{

if(Serial.available() > 0)       // Send data only when you receive data:

{

data = Serial.read();        //Read the incoming data and store it into variable data

Serial.print(data);          //Print Value inside data in Serial monitor

Serial.print("\n");          //New line

if(data == '1')              //Checks whether value of data is equal to 1

digitalWrite(13, HIGH);  //If value is 1 then LED turns ON

else if(data == '0')         //Checks whether value of data is equal to 0

digitalWrite(13, LOW);   //If value is 0 then LED turns OFF

}

}

8) Zigbee

Upload this code to your Arduino

# Code :-

int ledPin = 13;void setup() {

Serial.begin( 9600 );    //  baud rate 9600 for the serial Bluetooth communication

}void loop() {

// listen for the data from raspberry pi

if ( Serial.available() > 0 ) {

// read a numbers from serial port

int inputVal = Serial.parseInt();    if (inputVal > 0) {

Serial.print("Your input is: ");

Serial.println(String(inputVal));

// Here blink the LED

blinkLED(inputVal);

}

}

}void blinkLED(int inputVal) {

for (int i=0; i< inputVal; i++) {

digitalWrite(ledPin, HIGH);

delay(500);

digitalWrite(ledPin, LOW);

delay(500);

}

}